Legi-Air 4050B-BB

|  |  | Technical Specifications: |  |
| --- | --- | --- | --- |
|  |  | Printing Technology Print Resolution Print Speed | Thermal transfer or direct thermal |
|  |  |  | 8 dots / mm (203 dpi) 12 dots / mm (300 dpi) 24 dots / mm (600dpi) |
|  |  |  | Up to 400 mm per second |
|  |  | Label Size | Blow Standard from 50 x 40mm to 100 x 100mm, depending on label qualitiy |
|  |  | Tamp Sizes | - |
|  |  | Label Gap Label Roll | minimum 3 mm |
|  |  |  | 300 mm outside diameter up to 450 rm, core 76mm (3“), outside winding optional: inside winding |
|  |  | Data Interface | All current interfaces (details in prospect print engines) |
|  |  | Controller Interface | Volt free outputs (relays), opt coupler inputs, M12 connector for photocell and alarm lamp |
|  |  | Maintenance Interval | According to interval display |
|  |  | Performance Data | Up to 240 labels per minute, performance data depends on label size, text change |
|  |  | Application Accuracy | Standard: up to ± 1,5 mm on stopped product (depending on application distance, type of applicator and label size) |
|  |  | Print-Apply-Trigger | Reflex sensor, light barrier, inductive or capacitive sensor, micro switch, volt free contact (PLC) |
|  |  | Alarm System | Standard: LED error message in text display, volt free contact (PLC), optional: 3-coloured alarm lamp |
|  |  | Compressed Air Consumption Compressed Air Connection | 0,3 up to 1 l per applying cycle, depending on label size |
|  |  |  | 6 bar, Compressed air has to be dry and oil-free (DIN ISO 8573-1) |
|  |  | Weight | Up from 35 kg (depending on version) |
|  |  | Power Connection | 91 up to 263 VAC, 50/60 Hz, 5 Ampere |
|  |  | Environmental Conditions | Temperature: 10° C up to 35° C, 2 up to 95 % relative air humidity, non-condensing |
|  |  | Certification | CE-Declaration of Conformity |
|  |  | Options | 3-coloured alarm lamp IPC in protective housing LLS Legitronic® Labeling Software Bluhmware for controlling and networking the systems attendance check of labels Product sensors stands in different versions (fixed, adjustable via spindle, mobile) |
|  |  | Applicators | 4x 4” blow-box |
|  |  | Dimensions |  |
| Distribution arguments depending on system |  |  |  |
|  | - print engine technology of leading global manufacturers - no moving parts on applicator - Europe-wide distribution - very compact construction with a large spectrum of enhancement - developed for day and night operation - all components and also spare parts are standard and originated from large-volume production - simple and well-arranged system structure enables the customer`s technicians quick self-help measures in case of errors - large consultant and service network |  |  |

|  | Technical Specifications (in detail): |  |
| --- | --- | --- |
|  | Base Plate | 10 mm aluminum anodised |
|  | Housing of Applicator Controller | anodised aluminum profile |
|  | Housing of Print Engine Connections | - |
|  | Cabinet without fan | - |
|  | Festo Pneumatics | 0% |
|  | Gauge | - |
|  | Mechanical Components | metric |
|  | Modules / Replacement-Assemblies | standard |
|  | Display Lighting | aluminum anodised |
|  | (Remote) Display Remote mountable | standard |
|  | Display turnable | user-defined |
|  | Multilingual Display-Menu | EN, DE, FR, NL, NO, SE, RU, PL, ES, TR, PT, IT, CZ, HR, Optional: AZ, FI, HU, LV |
|  | Detailed Status and Error Messages | standard |
|  | Diagnostic Function Diagnosis Monitor | via display and USB |
|  | Adjustable cycle time of valves | in 1 ms steps |
|  | Service/Maintenance Counter Service Message | yes, display message + volt free contact |
|  | Variants of Applicator | 1 varia |
|  | Printer | flexible print engine technique: Datamax, Sato, Zebra, Avery, CAB |
|  | LH / RH | yes, at the same price |
|  | Max. Tamp (mm) | - |
|  | Max. Label (mm) | 100mm long or 100mm wide (preferred ratio w x l = 1,5-2:1) |
|  | Min. Tamp (mm) | - |
|  | Min. Label (mm) | 50 x 40 |
|  | Max. Stroke (useable) | - |
|  | Max. Stroke (useable; optional) | - |

|  | Recommended continuous power rating with max. stroke standard applicator PPM=products per minute |  | 3-shift, because of robust applicator construction no limitation for continuous operation |
| --- | --- | --- | --- |
|  | Top performance with max. stroke standard applicator |  | no difference to continuous rating |
|  | Top performance with KH High Speed Unit (small label, min. stroke) |  | - |
|  | Top performance with Blow Box |  | < 240 per min |
|  | Tolerance of Label Position (standard applicator) |  | ± 1.5mm |
|  | Tolerance of Label Position (precision applicator) |  | - |
|  | Tolerance of Label Position (Wipe-on mode) |  | - |
|  | Alarm Lamp M12 connector |  | standard |
|  | 1st Product Sensor M12 connector |  | standard |
|  | 2nd Product Sensor M12 connector |  | standard |
|  | Product Sensor configuration NPN/PNP |  | via display |
|  | Possibilities of configuration via PC |  | standard / USB |
|  | Signal exchange to customer via volt free contact |  | standard |
|  | Max. Roll Diameter |  | 300 mm |
|  | Max. Run Width |  | 160 mm optional: 173 mm |
|  | Unwind Discs |  | standard |
|  | Rewind (stepper motor technique) |  | standard |
|  | Quick Change |  | - |
|  | Variable Product Detection |  | - |
|  | Low Label Prewarning |  | option |
|  | Control Label on Tamp Pad |  | - |
|  | 2-level Vacuum |  | - |
|  | Service – Export Possibility |  | D, A, CH, B, DK, NL, F, E, I, P, UK, IRL, N, S |
|  | Possibility to save and recall label data in the labeler |  | - |
| Weiter Optionen: |  |  |  |